LiDAR-Based Tunnel Time-Series Shape Analysis Technology

SSL Smart Tunnel Monitoring System  |  Step 6 — Time-Series Analysis Module

# 1. Technical Overview and Necessity

Railway tunnels are long-lived linear structures in which shape changes — crown settlement, sidewall convergence, sectional eccentricity, and local damage — accumulate over years of operation. Conventional visual, tape, and total-station inspection is operator-dependent and typically captures only a single qualitative snapshot. What is needed is a time-series framework that quantitatively tracks how the tunnel shape evolves and objectively evaluates the location, magnitude, and rate of deformation.

Terrestrial LiDAR acquires dense 3D point clouds of the tunnel interior contact-free and at millimetre precision, even under live-traffic or maintenance conditions, making it an ideal input for digital-twin-based maintenance. The SSL Smart Tunnel Monitoring System is a fully automated Python pipeline that co-registers LiDAR scans from different epochs into a common reference frame, extracts shape change relative to the tunnel centreline and cross-sections, and combines M3C2-based 4D change detection with per-section engineering indices, producing report-ready outputs for digital-twin-based maintenance.

This technology corresponds to the mid-term consulting item “Intelligent Analysis–Simulation Technology 1: LiDAR-Based Tunnel Time-Series Shape Analysis” (DT 중간컨설팅_260707.pptx, slide 39) and is implemented as Step 6 of the tool, taking a T0 reference scan and Tn comparison scans — or a full T0~T5 multi-epoch set — as input.

# 2. Core Research Content

## 2.1 Multi-Point LiDAR Registration

Time-series comparison requires every epoch to share one reference coordinate frame; sensor pose, trajectory, and IMU error otherwise mask real structural change. T0 is fixed as the reference epoch and all other epochs are aligned to it.

Coarse alignment: feature/target-based initial transform (GROR + FPFH)

Fine registration: Generalized ICP (small_gicp), Open3D point-to-plane ICP as fallback

Benchmark: 0.196 mm RMSE, 339 ms (separate registration benchmark) — faster and more stable than Open3D ICP

Quality gate: registration RMSE and residuals checked; high RMSE flags results as low-confidence

## 2.2 Geometric Modeling and Shape Indices

The tunnel centreline is estimated and Frenet-frame cross-sections orthogonal to it are generated per chainage, which reduces apparent-ovality error on curved tunnels. Each section is compared with the design radius or the T0 reference section.

Shape indices: crown settlement, lateral convergence, ovality, eccentricity, local damage

Section fitting: Fitzgibbon Direct Least-Squares ellipse fitting for precise section geometry

Output: per-section tables and graphs; over-threshold zones highlighted in the 3D heatmap / 2D overlay

## 2.3 M3C2-Based 4D Change Detection and Trend Analysis

Using py4dgeo, T0 core points and local normals are fixed and the signed distance to each epoch is computed along the surface normal. A Level-of-Detection (LoD) criterion separates statistically significant deformation from noise, and results are aggregated by chainage, section angle, and epoch to form trends and a forecast.

Validation dataset: 80 m length, 3.0 m radius, 6 epochs (T0~T5), 15,456 points/epoch

Ground truth @ T5: crown -45 mm @20 m, convergence -35 mm @45 m, local damage -40 mm @65 m (from T3)

Measured @ T5: crown max 44.05 mm, heatmap max 45.0 mm, ovality 0.20 %, eccentricity 1.52 mm

Forecast: linear-trend R² ≈ 0.9997; caution at ~9 epochs, critical at ~15 epochs

# 3. Demo Video Walkthrough

The demo is a real screen recording of the SSL tool running the one-click Auto Pipeline on the validated T0~T5 dataset (file: docs/LiDAR_Tunnel_RealDemo_Annotated.mp4, ~1 minute, with on-screen step captions). Its purpose is to show how raw LiDAR point clouds become maintenance decision information, not to explain the algorithms. After the six epochs are loaded, a single “Auto Pipeline (1-click full analysis)” action runs the entire Step 1–Step 6 chain, and the following panels appear in sequence:

| Time | On-screen panel | What it demonstrates |
| --- | --- | --- |
| 0:03 | Auto Pipeline + Structure tree (S1–S6) | Six epochs T0~T5 loaded; one click runs preprocess → register → sections → M3C2. |
| 0:15 | 2D Cross-Section (circle fit) | Per-chainage section with crown/wall/floor deviation, ovality and eccentricity. |
| 0:28 | Multi-Epoch Deformation Trend | p95/median per epoch with Safe/Caution/Critical bands and a per-epoch status table. |
| 0:36 | M3C2 Deformation Map (T0→Tn) | Signed surface change with LoD; mm colorbar; results log of all extracted parameters. |
| 0:45 | Plot 2D Technical Section | Engineering section vs vehicle clearance; exportable to PDF / IFC. |
| 0:56 | Summary Dashboard | Critical zones, max deformation, trend forecast, and digital-twin handoff. |

# 4. Research Outcomes and Future Improvements

Outcomes

End-to-end automated pipeline reducing a manual multi-step workflow to a one-click run.

Centreline + Frenet sectioning with five engineering shape indices per section.

Benchmark-validated M3C2 tracking on T0~T5 (crown max 44.05 mm vs -45 mm ground truth; forecast R² ≈ 0.9997).

Full IFC4X3 export for BIM interoperability with digital-twin platforms.

Integrated RAG-LLM / AI Engineering Assistant for decision support.

Future Improvements

Robust validation on real field LiDAR (sensor drift, occlusion, reflectivity, cable/equipment clutter).

Multi-epoch UI workflow selecting T0~T5 at once with epoch-labelled graphs.

Separate cumulative (T0→Tn) and incremental (Tn→Tn+1) deformation in the report.

Link LoD significance to structural safety thresholds for clear caution/critical limits.

Non-circular profiles (horseshoe), fracture detection, and water-seepage mapping.

With these enhancements, this technology can become a core intelligent analysis–simulation module for railway-tunnel digital twins.