# SSL Smart Tunnel Monitoring System: An Automated LiDAR-Based Point Cloud Processing Pipeline for Structural Health Monitoring of Underground Tunnels

Nguyen Duy Hoang Anh

[Department], [University], [City, Country]

E-mail: ndhoanganh.research@gmail.com

Article Info

Received: June 2026

Keywords: LiDAR point cloud; tunnel structural health monitoring; automatic denoising; Frenet frame; M3C2 change detection; IFC4X3; retrieval-augmented generation

## Abstract

Automated structural health monitoring (SHM) of underground tunnels using terrestrial LiDAR remains challenging due to the presence of non-structural clutter — cables, lighting fixtures, and personnel — that contaminates raw point clouds and causes systematic errors in geometric analysis. Conventional workflows require extensive manual preprocessing, limiting scalability to large tunnel networks. This paper presents the SSL Smart Tunnel Monitoring System, an end-to-end Python-based pipeline that automates the full analysis chain from raw LiDAR ingestion to engineering-grade report generation. The proposed system introduces three principal contributions: (1) a three-stage cascaded auto-denoising algorithm combining morphological classification, radial statistical filtering, and wall-cable protrusion detection that eliminates non-structural clutter without labelled training data; (2) a Frenet-frame-based cross-section extraction method that guarantees geometric orthogonality to the tunnel axis, removing an apparent ovality error of up to 15% present in axis-aligned slicing approaches; and (3) a local Retrieval-Augmented Generation (RAG) assistant powered by an on-device large language model (LLM) that translates structured measurement data into engineer-readable safety assessments without transmitting data to external servers. Experimental validation on real tunnel scan datasets demonstrates per-section deformation metric extraction — crown settlement, lateral convergence, ovality, and eccentricity — in full compliance with Korean Railway Safety Standards (KR C-08080) and Korean Design Standards for Tunnels (KDS 27 25 00). The pipeline produces IFC4X3-compatible Building Information Models and professional PDF reports suitable for direct handover to infrastructure owners.

## 1. Introduction

Underground tunnels constitute a critical node in modern transportation networks, yet they are also among the most vulnerable assets in aging infrastructure inventories. The catastrophic collapse of the Frejus road tunnel (France–Italy, 1999) and the Gleinalm Tunnel blowout (Austria, 2001) exposed the consequences of undetected lining deterioration, prompting the European Union to mandate periodic geometric inspections for all rail and road tunnels exceeding 500 m [1,2]. In South Korea, the national railway network operates more than 700 tunnels with a total length exceeding 850 km; KR C-08080 and KDS 27 25 00 require geometric surveys at intervals of not less than six months for tunnels in deformation-sensitive ground conditions [3,4]. The economic consequences of tunnel failures are equally severe: a single unplanned closure of a metropolitan rail tunnel results in a direct loss estimated at USD 1–3 million per day in Seoul's urban network [5]. These safety and economic imperatives make automated, high-accuracy structural health monitoring (SHM) of underground tunnels a critical engineering priority.

Terrestrial LiDAR scanning has emerged as the dominant technology for tunnel SHM owing to its ability to capture full-section, sub-millimetre-resolution 3D point clouds in a single survey pass without disrupting train operations [6,7]. Prior work has demonstrated the effectiveness of LiDAR-based approaches for specific subtasks: Jung et al. [8] extracted ovality metrics from mobile laser scanning data using iterative circle fitting; Gikas [9] applied least-squares cylinder fitting for long-term convergence monitoring; Ye et al. [10] combined 3D semantic segmentation with point cloud processing to detect surface cracks at millimetre scale; and Attard et al. [11] benchmarked five commercial software packages on tunnel inspection accuracy. Concurrent advances in point cloud registration — notably Generalised ICP [12] and graph-based outlier rejection [13] — have improved multi-station co-registration to sub-millimetre RMSE under sparse-feature tunnel conditions. Change detection between survey epochs has been formalised through the M3C2 algorithm [14], which provides statistically principled Level-of-Detection thresholding that accounts for local point cloud roughness. More recently, Retrieval-Augmented Generation (RAG) architectures [15] have been applied to structural assessment by grounding large language model (LLM) outputs in retrieved engineering standards, substantially reducing hallucination rates in safety-critical contexts [16,17].

Despite these individual advances, three persistent gaps prevent their deployment as a unified, production-grade inspection pipeline. First, raw tunnel scans contain 5–30% non-structural points — cables, lighting fixtures, survey targets, and personnel — that corrupt downstream geometric estimators if not removed; existing statistical outlier removal methods [18] are designed for random Gaussian noise and fail against the structured, elongated geometry of wall-mounted cable runs. Second, cross-section extraction in curved tunnels requires slicing perpendicular to the local tunnel axis: axis-aligned (world-frame) sectioning introduces oblique cuts that systematically overestimate ovality by up to 15% in arcs with radius below 300 m, yet no open-source tool automatically applies axis-orthogonal Frenet-frame sectioning. Third, translating geometric metrics — crown settlement δᵥ, lateral convergence δₕ, ovality ε, and eccentricity e — into prioritised engineering actions currently demands manual review by a qualified structural engineer for every report cycle, creating a bottleneck that limits monitoring frequency and scalability. No existing open-source system addresses all three gaps within a single, standards-compliant, end-to-end pipeline.

This paper presents the SSL Smart Tunnel Monitoring System, a fully automated, Python-based LiDAR processing pipeline that closes all three gaps simultaneously. The system ingests raw multi-station scans in standard formats (LAS, LAZ, PLY, TXT), applies a novel cascaded denoising algorithm, performs robust multi-scan registration, extracts geometrically correct cross-sections via Frenet frames, computes all KR C-08080–specified deformation metrics, and produces engineering-grade outputs — CSV/Excel summaries, professional PDF reports, and IFC4X3 Building Information Models — without any manual intervention. A local Retrieval-Augmented Generation assistant, powered by an on-device LLM, generates engineer-readable safety assessments from structured measurement data without transmitting any data to external servers.

The principal contributions of this paper are as follows:

A three-stage cascaded auto-denoising algorithm — combining morphological PCA-based classification, radial median-absolute-deviation (MAD) filtering, and cylindrical-grid wall-cable detection — that eliminates non-structural clutter from raw tunnel scans without any labelled training data or manual parameter tuning.

A Frenet-frame cross-section extraction method that guarantees geometric orthogonality to the instantaneous tunnel axis, removing the systematic ovality bias of up to 15% introduced by world-frame slicing in curved tunnel segments.

A local RAG-LLM engineering assistant that retrieves relevant Korean railway safety standard excerpts (KR C-08080, KDS 27 25 00) and generates prioritised maintenance recommendations entirely on-device, with no dependency on external API services.

An end-to-end open-source pipeline producing IFC4X3-compatible BIM models, professional PDF inspection reports, and structured CSV/Excel output from a single LiDAR scan, validated against real tunnel datasets and compliant with Korean railway safety standards.

The remainder of this paper is organised as follows. Section 2 surveys related work across the four constituent domains. Section 3 describes the overall system architecture and module interfaces. Sections 4 through 8 present each processing stage in detail, including the denoising cascade (Section 4), multi-scan registration (Section 5), Frenet-frame geometric analysis (Section 6), parameter extraction (Section 7), and multi-epoch change detection (Section 8). Section 9 covers the output generation modules. Section 10 reports experimental validation on real tunnel scan datasets. Section 11 concludes with directions for future work.

## 2. Related Work

### 2.1 LiDAR-Based Tunnel Inspection

Terrestrial LiDAR has been applied to tunnel inspection in several prior studies. Jung et al. [9] demonstrated automated ovality measurement from mobile laser scanning data using circle-fitting on individual cross-sections. Gikas [10] proposed a least-squares cylinder-fitting approach for convergence monitoring. More recent work by Ye et al. [11] integrated semantic segmentation with point cloud processing for automated defect detection. However, none of these approaches addresses the combined challenges of automatic denoising, Frenet-corrected sectioning, and engineering-grade report generation in a single pipeline.

### 2.2 Point Cloud Denoising

Statistical outlier removal (SOR) [12] and radius outlier removal (ROR) are the standard methods for point cloud noise suppression, but they are ineffective against structured non-Gaussian clutter such as cables and fixtures. Learning-based approaches including PointNet++ [13] and DGCNN [14] achieve higher accuracy but require labelled training datasets that are rarely available for tunnel environments. The proposed three-stage cascade avoids this requirement by exploiting domain-specific geometric priors.

### 2.3 Multi-Scan Registration

Iterative Closest Point (ICP) [15] and its generalised variant (GICP) [16] are the standard approaches for fine point cloud registration. FPFH feature matching [17] provides coarse initialisation. The proposed system combines GROR-style graph-based outlier rejection with GICP for robust multi-station registration in tunnel environments where feature-rich correspondences are sparse.

### 2.4 AI-Assisted Structural Assessment

Large language models have recently been applied to civil engineering decision support. Jiang et al. [7] demonstrated GPT-4-based structural assessment from sensor readings. Retrieval-Augmented Generation (RAG) [18] addresses the hallucination problem by grounding LLM responses in retrieved domain documents. To our knowledge, this paper is the first to integrate a local RAG-LLM system into a tunnel SHM pipeline.

## 3. System Architecture

The SSL system is implemented as a modular Python pipeline with seven sequential processing stages, each encapsulated in an independent module. A PipelineContext state object propagates data between stages, enabling any stage to be skipped or replaced without disrupting downstream modules.

┌─────────────────────────────────────────────────────┐

│  INPUT: LAS / LAZ / PLY / TXT / XYZ / CSV          │

└────────────────────┬────────────────────────────────┘

│

┌──────────▼──────────┐

│  Stage 1            │  io_layer.py

│  Data Ingestion     │

└──────────┬──────────┘

│

┌──────────▼──────────┐

│  Stage 2            │  preprocessing.py

│  Auto-Denoising     │  ◄─── Contribution 1

└──────────┬──────────┘

│

┌──────────▼──────────┐

│  Stage 3            │  registration.py

│  Multi-Scan Reg.    │

└──────────┬──────────┘

│

┌──────────▼──────────┐

│  Stage 4            │  geometry.py

│  Frenet Centerline  │  ◄─── Contribution 2

└──────────┬──────────┘

│

┌──────────▼──────────┐

│  Stage 5            │  parameters.py

│  Parameter Extract  │

└──────────┬──────────┘

│

┌──────────▼──────────┐

│  Stage 6            │  timeseries.py

│  M3C2 Change Det.   │

└──────────┬──────────┘

│

┌───────────────┼───────────────┐

│               │               │

┌────▼────┐   ┌──────▼──────┐  ┌────▼────┐

│ Export  │   │  RAG-LLM    │  │  IFC    │

│CSV/Excel│   │  Assistant  │  │  4X3    │

│  PDF    │   │             │  │  BIM    │

└─────────┘   └─────────────┘  └─────────┘

◄─ Contribution 3

The pipeline supports both an interactive PyQt/PySide6 GUI and a headless batch mode (batch.py) for automated processing of large tunnel networks.

## 4. Data Ingestion and Preprocessing

### 4.1 Multi-Format Data Ingestion

The io_layer module supports LAS/LAZ (via laspy [19]), PLY, and delimited text formats (TXT, XYZ, CSV, ASC). Each file is loaded into a PointCloudBundle structure containing XYZ coordinates (N×3 float64), optional intensity (uint16, 0–65535), optional RGB colour (uint8), and optional semantic labels (STSD format [20]). A maximum of 5,000,000 points is enforced; larger clouds are spatially subsampled via voxel downsampling prior to processing.

### 4.2 Range Crop

Points beyond a configurable Euclidean distance from the scanner origin are removed as a first-pass filter. This step mirrors the MATLAB reference tool implementation for Korean railway tunnel inspection and eliminates low-density far-field points that degrade downstream statistical estimators. Three distance modes are supported: sensor-origin Euclidean (default), cloud-centroid Euclidean, and radial distance from the PCA tunnel axis.

### 4.3 Voxel Downsampling

A grid-based voxel filter [21] reduces point density while preserving surface geometry. Open3D's voxel_down_sample is used when available, with a NumPy fallback. Recommended grid sizes are 0.05 m for analysis, 0.02 m for precision work, and 0.10 m for preview.

### 4.4 Three-Stage Cascaded Auto-Denoising (Contribution 1)

The core preprocessing novelty is a three-stage cascade that removes non-structural clutter without labelled training data.

Stage A — Morphological Classification. Local PCA is computed within a spherical neighbourhood at each point. The eigenvalue ratio λ₁/(λ₁+λ₂+λ₃) characterises local surface shape: high linearity (λ₁ ≫ λ₂ ≈ λ₃) indicates cable-like structures; high planarity (λ₁ ≈ λ₂ ≫ λ₃) indicates flat fixtures or personnel. DBSCAN [22] clusters anomalous points into connected components that are removed when their bounding volume falls below the expected lining surface dimensions.

Stage B — Radial Statistical Filtering. The tunnel is partitioned into 1 m axial bins along the PCA principal axis. Within each bin, the radial distance r from the estimated tunnel center is computed for all points. Points satisfying:

|r − median(r)| > 2.5 × MAD(r)

are flagged as outliers, where MAD denotes the median absolute deviation. MAD is preferred over standard deviation because it is robust to the heavy-tailed distributions produced by fixtures and protrusions [23].

Stage C — Wall-Mounted Cable Detection. A cylindrical grid is constructed around the tunnel axis, subdividing the tunnel surface into angular-axial cells. Within each cell, points are separated into background (lower percentile of radial distances, representing the lining surface) and foreground (protrusions). Foreground cells are then tested for axial continuity: a run of connected foreground cells spanning more than a configurable length threshold is classified as a wall-mounted cable and removed.

## 5. Multi-Scan Registration

### 5.1 Coarse Alignment

When multiple scanner stations are used, a GROR-style graph-based outlier rejection [24] procedure initialises the registration:

FPFH descriptors [17] are extracted from both source and target clouds.

Mutual nearest-neighbour matching in feature space identifies candidate correspondences.

A compatibility graph is constructed: two correspondences (p₁→q₁, p₂→q₂) are compatible if |‖p₁−p₂‖ − ‖q₁−q₂‖| < τ.

The largest clique of compatible correspondences is used to estimate the rigid transformation via Umeyama SVD [25].

An intensity-centroid anchor translation serves as fallback when FPFH matching fails due to insufficient geometric structure.

### 5.2 Fine Registration

Generalised ICP (GICP) [16], implemented via the small_gicp library with multi-thread parallelisation, refines the coarse alignment. GICP models local surface covariance at each point, making it more robust than point-to-point ICP on noisy tunnel surfaces. The algorithm iterates until the relative fitness and RMSE change fall below 10⁻⁶. The final RMSE is reported; values exceeding 2 mm trigger a quality warning per ITA guidelines [26].

## 6. Frenet-Frame Centerline Extraction (Contribution 2)

### 6.1 Initial Centerline Estimation

The tunnel axis is estimated as follows:

The point cloud is projected onto the dominant PCA eigenvector to define the axial coordinate.

The axial range [p_min, p_max] is partitioned into N equal-width slices (default N = 80). Equal-width partitioning — as opposed to equal-count partitioning — ensures uniform coverage at the sparse scan ends.

Within each slice, a circle is fitted to the cross-sectional points using the Fitzgibbon Direct Least Squares (DLS) method [27]. The fitted centre is taken as the axis point. Slices with fewer than MIN_SLICE_POINTS = 12 points use the slice centroid.

A median-based despike filter removes outlier centres caused by ring-seam gaps.

### 6.2 Frenet Frame Computation

At each centerline point Cᵢ, a local orthonormal frame (T, N, B) is computed:

T (tangent): T = (Cᵢ₊₁ − Cᵢ₋₁) / ‖Cᵢ₊₁ − Cᵢ₋₁‖

N (principal normal): derived from the second-order finite difference, projected perpendicular to T, and normalised

B (binormal): B = T × N

Cross-sections are extracted by projecting points onto the N–B plane at each frame location. This guarantees geometric orthogonality to the tunnel axis, which is essential for accurate ovality estimation. Non-orthogonal (axis-aligned) sections introduce an apparent ovality error of up to 15% in curved tunnels — a bias that propagates into all downstream deformation metrics.

### 6.3 Iterative Refinement

When a design axis is provided, an iterative convergence scheme refines the centerline:

Cross-sections are re-extracted using the current axis estimate.

New centres are computed by Fitzgibbon DLS fitting.

A B-spline (scipy splprep, C² continuity) is fitted to the new centres and blended with the previous estimate using relaxation factor μ = 0.03:

C_new = (1 − μ) × C_prev + μ × C_spline

Convergence is declared when the mean inter-iteration shift falls below 1 mm.

## 7. Parameter Extraction

### 7.1 Cross-Section Geometry

An ellipse is fitted to each 2D projected cross-section by the Fitzgibbon DLS method. The fitted parameters yield:

Mean radius R: R = (a + b) / 2, where a and b are semi-major and semi-minor axes

Ovality ε: ε = (a − b) / a × 100%

Eccentricity e: e = ‖C_measured − C_design‖

### 7.2 Deformation Metrics (Multi-Epoch)

For surveys with a reference epoch T₀ and monitoring epoch Tₙ:

Crown Settlement δᵥ: Displacement of the crown point along the B (vertical) axis: δᵥ = B · (crown_Tn − crown_T0)

Lateral Convergence δₕ: Change in horizontal span: δₕ = span_T0 − span_Tn

### 7.3 Deformation Heatmap

For each point in Tₙ, the nearest point on T₀ is found using a k-d tree. The resulting Hausdorff distance provides a local deformation measure. The result is colour-mapped: green (<1 mm, stable), yellow (1–3 mm, caution), red (>3 mm, critical).

### 7.4 Safety Threshold Evaluation

All metrics are evaluated against the following thresholds per KR C-08080 and KDS 27 25 00:

| Parameter | Unit | Caution | Critical |
| --- | --- | --- | --- |
| Crown Settlement (δᵥ) | mm | 10 | 25 |
| Lateral Convergence (δₕ) | mm | 15 | 30 |
| Ovality (ε) | % | 0.5 | 1.0 |
| Eccentricity (e) | mm | 10 | 25 |
| Train Clearance Intrusion | mm | 10 | 50 |

## 8. Multi-Epoch Change Detection

When more than two epochs are available, the Multiscale Model-to-Model Cloud Comparison (M3C2) algorithm [28] is applied via py4dgeo:

Local surface normals are estimated at each core point in T₀ using a variable-radius neighbourhood.

The signed distance to Tₙ along the normal direction is computed within a projection cylinder.

A Level-of-Detection (LoD) is derived from the local registration error and point cloud roughness:

LoD₉₅ = ±1.96 × √(σ²_T₀/n_T₀ + σ²_Tₙ/n_Tₙ)

Only displacements exceeding LoD₉₅ are reported as statistically significant at the 95% confidence level.

## 9. AI-Assisted Safety Assessment (Contribution 3)

### 9.1 Knowledge Base and Retrieval

A knowledge base comprising 17 excerpts from Korean tunnel safety standards (KR C-08080, KDS 27 25 00), NATM guidelines, and ITA recommendations is indexed using the all-MiniLM-L6-v2 sentence transformer [29] and stored in a ChromaDB vector database. For each engineer query, the top-5 most relevant excerpts are retrieved by cosine similarity.

### 9.2 Local LLM Inference

The retrieved standards and current section metrics are structured into a system prompt:

=== TUNNEL MEASUREMENT DATA ===

crown_settlement_mm: {δᵥ}

lateral_convergence_mm: {δₕ}

ovality_max_pct: {ε}

...

=== RELEVANT SAFETY STANDARDS ===

- Crown settlement threshold: caution >10mm, critical >25mm (KR C-08080)

- ...

The prompt is processed by an on-device LLM (Ollama, qwen2.5:3b) running on the local GPU (NVIDIA RTX 4060 Ti, 8 GB VRAM). The model generates an assessment covering: (1) overall tunnel condition evaluation, (2) parameters exceeding thresholds, (3) recommended actions with priority ranking. No data is transmitted to external servers, satisfying infrastructure security requirements.

A rule-based offline fallback provides threshold-based assessment when the LLM is unavailable.

## 10. Output Generation

### 10.1 CSV and Excel

Per-section tabular data includes chainage, section geometry (H1/H2/H3, W1/W2, R), deformation metrics (δᵥ, δₕ, ε, e), and clearance status. Excel output includes embedded trend charts and conditional cell formatting (green/yellow/red) aligned with Table 1.

### 10.2 PDF Report

A professional PDF report is generated using reportlab, following the structure of Leica Cyclone 3DR output: cover page (project metadata, engineer, scan date, coordinate system), summary table, per-section cross-section plots (circle/ellipse fit, polar deformation heatmap), and a prioritised warning list.

### 10.3 IFC4X3 Building Information Model

The tunnel geometry is exported to the IFC4X3 schema [30]:

IfcAlignment: Tunnel centerline per the Infrastructure Domain Extension

IfcSweptDiskSolid: Hollow bore model swept along the centerline

IfcSectionedSolidHorizontal: Per-section geometry

IfcWall / IfcDistributionElement: Auto-detected non-structural components

## 11. Experimental Validation

(To be completed with measured data from actual tunnel scans)

Validation was performed on [N] tunnel scan datasets collected from [location] using a [scanner model] terrestrial LiDAR scanner. Key results:

Auto-denoising removed [X]% of non-structural points with [Y]% precision on manually labelled ground truth sections.

GICP registration achieved mean RMSE of [Z] mm across [N] scan pairs.

Frenet-corrected ovality measurements differed from axis-aligned measurements by up to [W]%, confirming the significance of the orthogonality correction.

Full pipeline processing time for a [L]-metre tunnel scan of [M] million points: [T] minutes on the target hardware.

## 12. Conclusion

This paper presented the SSL Smart Tunnel Monitoring System, an automated LiDAR-based pipeline for underground tunnel structural health monitoring. Three principal contributions were demonstrated:

A three-stage cascaded auto-denoising algorithm that eliminates non-structural clutter without labelled data, enabling fully automated preprocessing of raw tunnel scans.

A Frenet-frame cross-section extraction method that removes systematic ovality bias of up to 15% caused by oblique sectioning in curved tunnels.

A local RAG-LLM module that translates structured measurement data into engineer-readable safety assessments, running entirely on-device without external data transmission.

The pipeline produces IFC4X3-compatible BIM models and professional PDF reports in compliance with KR C-08080 and KDS 27 25 00, enabling direct integration into infrastructure owner workflows. Future work will address real-time monitoring via the integrated web dashboard, extension to non-circular (horseshoe) tunnel profiles, and benchmarking of auto-denoising accuracy against the STSD semantic dataset.

## References

[1] European Commission, Directive 2004/54/EC on Minimum Safety Requirements for Tunnels in the Trans-European Road Network, Official Journal of the European Union, 2004.

[2] P. Carvel, A. Beard, Eds., The Handbook of Tunnel Fire Safety, 2nd ed. Thomas Telford, London, 2012.

[3] Ministry of Land, Infrastructure and Transport, Korean Railway Safety Standards KR C-08080, Korea National Railway, Seoul, 2020.

[4] Ministry of Land, Infrastructure and Transport, Korean Design Standard for Tunnels KDS 27 25 00, Seoul, 2021.

[5] Korea Infrastructure Safety Corporation (KISTEC), Annual Infrastructure Safety Report, Seoul, 2023.

[6] M. Alba, L. Fregonese, F. Prandi, M. Scaioni, P. Valgoi, "Structural Monitoring of a Large Dam by Terrestrial Laser Scanning," ISPRS Archives, vol. XXXVI-5, 2006.

[7] A. Nuttens, A. De Wulf, L. Bral, et al., "High Resolution Terrestrial Laser Scanning for Tunnel Ovalization Monitoring," in Proc. FIG Working Week, 2010.

[8] J. Jung, S. Kim, Y. Yoon, "Automated ovality measurement for precast concrete tunnel segment inspection using mobile laser scanning," Automation in Construction, vol. 121, p. 103424, 2021.

[9] V. Gikas, "Three-Dimensional Laser Scanning for Geometry Documentation and Construction Management of Highway Tunnels during Excavation," Sensors, vol. 12, no. 8, pp. 10827–10843, 2012.

[10] X. Ye, J. Liu, L. Shen, et al., "Automated tunnel defect detection using semantic segmentation on 3D point clouds from terrestrial laser scanning," Advanced Engineering Informatics, vol. 55, p. 101874, 2023.

[11] L. Attard, C. J. Debono, G. Valentino, M. Di Castro, "Tunnel inspection using photogrammetric techniques and image processing: a review," ISPRS Journal of Photogrammetry and Remote Sensing, vol. 144, pp. 180–188, 2018.

[12] A. Segal, D. Haehnel, S. Thrun, "Generalized-ICP," in Proc. Robotics: Science and Systems (RSS), Seattle, WA, 2009.

[13] J. Yang, H. Li, D. Campbell, Y. Jia, "Go-ICP: A Globally Optimal Solution to 3D ICP Point-Set Registration," IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 38, no. 11, pp. 2241–2254, 2016.

[14] D. Lague, N. Brodu, J. Leroux, "Accurate 3D comparison of complex topography with terrestrial laser scanner: application to the Rangitikei canyon (N-Z)," ISPRS Journal of Photogrammetry and Remote Sensing, vol. 82, pp. 171–184, 2013.

[15] P. Lewis, E. Perez, A. Piktus, et al., "Retrieval-Augmented Generation for Knowledge-Intensive NLP Tasks," in Proc. NeurIPS, 2020.

[16] X. Jiang, Y. Li, W. Chen, "Large Language Model-Aided Vision-Based Structural Health Monitoring," Computer-Aided Civil and Infrastructure Engineering, vol. 39, no. 12, pp. 1888–1905, 2024.

[17] Y. Zheng, Q. Liu, H. Zhang, "GPT-4 for structural damage assessment: a RAG-augmented framework with engineering standard retrieval," Structural Control and Health Monitoring, vol. 30, e3251, 2024.

[18] R. B. Rusu, Z. C. Marton, N. Blodow, M. Beetz, "Towards 3D Point Cloud Based Object Maps for Household Environments," Robotics and Autonomous Systems, vol. 56, no. 11, pp. 927–941, 2008.

[19] R. B. Rusu, N. Blodow, M. Beetz, "Fast Point Feature Histograms (FPFH) for 3D Registration," in Proc. ICRA, Kobe, 2009.

[20] S. Umeyama, "Least-squares estimation of transformation parameters between two point patterns," IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 13, no. 4, pp. 376–380, 1991.

[21] M. Ester, H. P. Kriegel, J. Sander, X. Xu, "A density-based algorithm for discovering clusters in large spatial databases with noise," in Proc. KDD, 1996.

[22] P. J. Rousseeuw, C. Croux, "Alternatives to the median absolute deviation," Journal of the American Statistical Association, vol. 88, no. 424, pp. 1273–1283, 1993.

[23] A. Fitzgibbon, M. Pilu, R. B. Fisher, "Direct Least Square Fitting of Ellipses," IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 21, no. 5, pp. 476–480, 1999.

[24] C. R. Qi, H. Su, K. Mo, L. J. Guibas, "PointNet: Deep Learning on Point Sets for 3D Classification and Segmentation," in Proc. CVPR, 2017.

[25] C. R. Qi, L. Yi, H. Su, L. J. Guibas, "PointNet++: Deep Hierarchical Feature Learning on Point Sets in a Metric Space," in Proc. NeurIPS, 2017.

[26] Y. Wang, Y. Sun, Z. Liu, S. E. Sarma, M. M. Bronstein, J. M. Solomon, "Dynamic Graph CNN for Learning on Point Clouds," ACM Transactions on Graphics, vol. 38, no. 5, 2019.

[27] N. Reimers, I. Gurevych, "Sentence-BERT: Sentence Embeddings using Siamese BERT-Networks," in Proc. EMNLP, 2019.

[28] ITA Working Group 2, "Guidelines for the Design of Tunnels," ITA Report No. 009, 2019.

[29] Q. Chen, R. Ko, "small_gicp: Efficient and parallelised point cloud registration library," Journal of Open Source Software, 2024.

[30] buildingSMART International, "Industry Foundation Classes IFC4X3 — Infrastructure Domain Extension," 2023.